# TAPI ClientTest

## Abstract

The TAPI test, tapiclient.dll, verifies that peripheral supports Telephony API (TAPI) functionality, especially the ability to complete outbound calls.

This test repeatedly sets up and dials a call on a TAPI-enabled device. It does not attempt any communication beyond simple device connection. It may be used as a build verification test (BVT); as a breadth test, by varying command-line settings; and as a stress test, by setting it to dial multiple times.

## Hardware Requirements

| Requirements | Description |
| --- | --- |
| A TAPI device | External modem, PCMCIA modem, null modem, or IR modem. |
| A RAS server | Any RAS server to be dialed. The TapiServer test can also be used. |

## Required Software

| tapiclient.dll | Test dll file. |
| --- | --- |
| Common library files | Located in the Netlib directory. |

## Running the Test

The following command line shows the syntax for running the test.

tux –o –d tapiclient –c “[parameters]”

tux -o -d tapiclient -c "-d1 –pT9,555-1212 -n1" -x 200,201

The following table shows the general test case parameters.

| Parameter | Description |
| --- | --- |
| -l | Lists all TAPI devices, then exits without making any calls. |
| -n number_of_calls | Sets the number of calls to be made. Default:1. |
| -p “phone_number” | Sets the telephone number to be dialed. Default: # (NULL). The number may be specified using any one of three formats: - If the number begins with “+”, it will be read as being in canonical format (“+country code (area code) local number”) and translated according to TAPI dialing rules for the current location. Example: -p“+1 (425) 555-1212” - If the number begins with “#”, the telephone number will be set to NULL. This is acceptable in some scenarios, such as when testing a null modem. - If the number begins with any other character, it will be dialed directly. Dialable numbers should be preceded with “P” for pulse or “T” for tone dialing plus any digits that may be needed to access an outside line. Example: -pT9,555-1212 |
| -d device_identifier | Sets the TAPI device identifier of the line to be tested. A list of device identifiers may be obtained by using the –l parameter. The default value is 0, which is the external modem on most standard Windows CE configurations. |
| -td seconds | Sets the dialing timeout, in seconds. A call will be dropped if it has not completed by this time. The default value is 30 seconds for Windows CE, which waits for a call to connect before completing it, and 5 seconds for Windows NT, which completes first and then connects. |
| -tc seconds | Sets the connection delay, in seconds. The program will wait as long as this interval after a call completes before dropping it. The default value is 10 seconds for Windows CE, 30 seconds for Windows NT. |
| -th seconds | Sets the hang-up command timeout interval in seconds. Default: 10. The program will execute for as long as this interval before timing out. |
| -tw seconds | Sets the deallocation delay interval in seconds. Default: 1. The program will wait for as long as this delay interval after a call has been dropped before deallocating the call handle. |
| -r baud | Sets the baud. Windows CE currently ignores this parameter and sets the baud from the device configuration instead. Default: 19200. |
| -e | Allows you to edit device configuration and current location. Two dialog boxes are opened: - Device Properties Allows the baud and other parameters for the TAPI device to be changed. - Dialing Properties Allows the location and other parameters affecting address translation to be changed. |
| -h | Prints a short help description. |

The following are additional test features:

- Testing of null modem serial connections.

In order for a null modem connection to complete, the bearer mode must be set to 3 (data transfer) and the baud parameters in the Device Properties dialog box must be set correctly.

- If the logging verbosity is maximized (-v15 parameter), data from the LINEDEVCAPS structure for the device being tested will be logged.

This provides important information, such as the name of the device and the bearer, and the media modes it supports.

- Hayes AT modem commands may be entered using the Extra Settings box in the Device Properties dialog box.

Commands should be listed without the leading “AT” string. For example, to turn off the modem speaker (Hayes command = ATM0), enter “M0”.

## Test Cases

| Test Case ID | 100: List All TAPI Device Names |
| --- | --- |
| Test Description | This test lists all TAPI devices, then exits without making any calls. |
| Troubleshooting | This test will fail if the application, TAPI, or service-provider versions are incompatible. |

| Test Case ID | 200: VOICE – DATAMODEM Test |
| --- | --- |
| Test Description | This test case sets the bearer mode to LINEBEARERMODE_VOICE and the media mode to LINEMEDIAMODE_DATAMODEM |
| Troubleshooting | This test will fail if the service provider does not support 3.1 kHz analog voice as bearer mode and data modem as media mode. |

| Test Case ID | 201: DATA – DATAMODEM Test |
| --- | --- |
| Test Description | This test case sets the bearer mode to LINEBEARERMODE_DATA and the media mode to LINEMEDIAMODE_DATAMODEM |
| Troubleshooting | This test will fail if the service provider does not support data transfer as bearer mode and data modem as media mode. |